!!! Important to note !!! If data is not available or a specific answer is difficult – note an assumption and/or a range in which the data could be located. Please also leave a comment on which source your answer is based on (e.g. research paper, personal measurement, engine manual, assumption etc.). If your source is an assumption, please note on what it is based (e.g.visual observation, comparable process, etc.). If you have any attachments, please note them in the source.

Please do not leave any empty space. An assumption is always better then nothing.

SFR

Topic 1: Machine Production:

1. What is the weight of the project SFR Unit?

If it is possible also estimate the weight of different machine sizes

| Kind of machine | Weight [kg] | Source |
| --- | --- | --- |
| Project Machine | App. 12,000 | Estimated based on our own design for a plant with a capacity of 240 m³/h of raw biogas. The weight does not include the co-generator. The design assumes the plant will be housed in a 40-foot container. |
| e.g. small | 7,800 | The data were obtained based on an estimation of the weight of components that vary depending on the installation’s capacity. |
| e.g. medium | 12,000 | The data were obtained based on an estimation of the weight of components that vary depending on the installation’s capacity. |
| e.g. large | 18,000 | The data were obtained based on an estimation of the weight of components that vary depending on the installation’s capacity. |

2. What materials are used in the machine and in what proportions (if no data is available, estimates will be okay)?

| Kind of Material | Share of total machine weight [%] | Source |
| --- | --- | --- |
| AISI 316 | 50 | Estimated based on our own design |
| ASTM A572 Grade 50 | 42 | Estimated based on our own design |
| Aluminum | 2.1 | Pumps, electric motors, plate heat exchangers, control and measurement devices |
| Plastics | 0.6 |  |
| Cooper | 2.3 |  |
| Others | 5.0 |  |

3. Where is the machine manufactured?

| Name of Company | City, Country | Source |
| --- | --- | --- |
| The company will be selected in a public tender |  |  |

4. What is the average lifespan of the machine?

| Lifespan [a] | Source |
| --- | --- |
| The system has many pieces of equipment that require maintenance and repair. I estimate that the pumps and blower will require maintenance every 1-2 years. The remaining equipment will likely have a lifespan of 3-5 years, and the SFR system itself will last a lifetime. | Estimated based on the operation of a pilot plant for agricultural biogas upgrading |

5. Are there any other factors that you consider as important?

Topic 2: Process parameters:

1. What is the desired CH4 content in the Output gas?

The output gas has a CH4 content of >=45 [%]

2. What is the maximum machine throughput at different LFG CH4 contents for aiming the desired CH4 content of the output gas?

| CH4 [%] | Throughput [L of LFG/h] | Source |
| --- | --- | --- |
| 5 | 31 250 |  |
| 10 | 35 714 |  |
| 15 | 41 667 |  |
| 20 | 50 000 |  |
| 30 | 83 333 |  |
| 35 | 125 000 |  |
| 40 | 250 000 |  |

2. Which inputs are required for the operation of the machine?

Input

| Electricity | 1,23E-01 | kWh/m3 raw biogas (45% CO2) |
| --- | --- | --- |
| Demineralized water | 1,04E-02 | dm3/m3 raw biogas (45% CO2) |
| Amine | 7,60E-04 | dm3/ m3 raw biogas (45% CO2) |
| Thermal oil | 4,77E-05 | dm3/ m3 raw biogas (45% CO2) |
| Ethylene glycol | 2,34E-05 | dm3/ m3 raw biogas (45% CO2) |
| Heat | 7,58E-01 | MJ/ m3 raw biogas (45% CO2) |

3. Define the quantity of Inputs needed at different LFG CH4 contents at maximum throghput:

| CH4 [%] | Amine[L/h] | Electricity [kWh] | Heat kWh | Thermal Oil [L/h] | Demineralized water [L/h] | Ethylene glycol [L/h] |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 5,21E-2 | 1,52 | 9,36 | 0,002 | 0,13 | 0,002 |
| 10 | 5,21E-2 | 1,33 | 8,19 | 0,002 | 0,11 | 0,002 |
| 15 | 5,21E-2 | 1,14 | 7,02 | 0,002 | 0,10 | 0,002 |
| 20 | 5,21E-2 | 0,95 | 5,85 | 0,002 | 0,08 | 0,002 |
| 30 | 5,21E-2 | 0,57 | 3,51 | 0,002 | 0,05 | 0,002 |
| 35 | 5,21E-2 | 0,38 | 2,34 | 0,002 | 0,03 | 0,002 |
| 40 | 5,21E-2 | 0,19 | 1,17 | 0,002 | 0,02 | 0,002 |

3. At a constant CH4 content – How does the throughput influence the consumption of operating resouceces?

| Throughput [% of maximum] | Amine[L/h] | Electricity [kWh] | Heat kWh | Thermal Oil [L/h] | Demineralized water [L/h] | Ethylene glycol [L/h] |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 5,21E-2 | 0,617 | 3,792 | 0,002 | 0,052 | 0,002 |
| 20 | 5,21E-2 | 1,234 | 7,583 | 0,002 | 0,104 | 0,002 |
| 30 | 5,21E-2 | 1,850 | 11,375 | 0,002 | 0,156 | 0,002 |
| 40 | 5,21E-2 | 2,467 | 15,167 | 0,002 | 0,208 | 0,002 |
| 50 | 5,21E-2 | 3,084 | 18,958 | 0,002 | 0,260 | 0,002 |
| 60 | 5,21E-2 | 3,701 | 22,750 | 0,002 | 0,313 | 0,002 |
| 70 | 5,21E-2 | 4,317 | 26,542 | 0,002 | 0,365 | 0,002 |
| 80 | 5,21E-2 | 4,934 | 30,333 | 0,002 | 0,417 | 0,002 |
| 90 | 5,21E-2 | 5,551 | 34,125 | 0,002 | 0,469 | 0,002 |
| 100 | 5,21E-2 | 0,617 | 3,79E+01 | 0,002 | 0,052 | 0,002 |

3. What maintenance work do you expect at an average machine utilization and in what time intervals?

| Kind of maintenance | Replacement interval [h] | Estimated work time [h] | Source |
| --- | --- | --- | --- |
| thermal oil replacement | 17600 | 8 | Estimated based on our own design |
| glycol replacement | 17600 | 8 | Estimated based on our own design |
| amine replacement | 26400 | 8 | Estimated based on our own design |
| seal replacement | 17600 | 24 | Estimated based on our own design |
| Pumps and funs maintenance | 8800 | 16 | Estimated based on our own design |

5. Which parameters can you identify to impact the maintenance efforts?

| Parameters | Strong, medium, lwo | Source |
| --- | --- | --- |
| Presence of O2 in biogas | Strong | General information about amine washing |

6. Is pretreatment for the LFG necessary? If yes what kind of?

| Pretreatment necessary [Yes/No] | Kind of pretreatment | Source |
| --- | --- | --- |
| Yes | Removal of H2S from biogas |  |

6. Are there any other aspects that you consider as important for the engine operation?

Topic 3: Solvent

1. Type of Solvent

| Solvent | Abb. | CAS Number |
| --- | --- | --- |
| 2-amino-2-methyl-1-propanol | AMP | 124-68-5 |
| 2-(Ethylamino)ethanol | EAE | 110-73-6 |
| 2-(Dimethylamino)ethanol | DMAE | 108-01-0 |
| 2-Methylpiperazine | 2-MPZ | 109-07-9 |
| Diethylenetriamine | DETA | 111-40-0 |

2. Which raw materials are needed to produce 1 Liter of solvent

| Kind of raw material | quantity | Source |
| --- | --- | --- |
| Crude oil |  |  |
| Ammonia (air) |  |  |
| Water |  |  |

3. Which machines are used to produce the solvent?

| Kind of machine | Process step | Source |
| --- | --- | --- |
| Refinery | Distillation, Extraction, Hydrocracking, desulfurization, reforming, etc. |  |
| Nitrogen Plant | Habera–Boscha process |  |

4.  How much energy is needed to produce 1 Liter of solvent?

| Type of Energy | kWh for 1l of Diesel | Source |
| --- | --- | --- |
| e.g. thermal | 37-75 MJ/L | https://support.ecoinvent.org/impact-assessment?utm_source=chatgpt.com |
| e.g. electrical | No data available |  |

5. Are there any other factors that you consider as important?

Topic 4: Output Parameters

1. What type of emissions do you exspect?

Name of Emission

Solvent waste

CO2 captured

3. Please fill out the followng table on the solvent consumption

| Initial solvent charge (first fill) | 1.5 m3 | m³ or kg | Quantity required for initial operation |
| --- | --- | --- | --- |
| Replacement frequency | ☐ Every year X Every 3 years ☐ >5 years ☐ Other: ______ | – | Based on degradation or performance drop |
| Average solvent loss per year | 50% | % of initial charge or kg/year | Includes degradation + operational loss |
| Is the solvent regenerated? If yes, with which efficiency? | No | % | How much solvent recovered from regeneration |
| How much energy is needed to recover 1L of solvent | - | kWh |  |
| Disposal of Solvent: |  |  |  |
| Hazardous classification | ☐ Yes07 01 08* – Other waste containing hazardous substances | – | UN number, hazard class |
| Safety measures in handling | Transport: according to ADR, class 8 (corrosive substances). Handle amines only in well-ventilated areas or under a fume hood to prevent inhalation of vapors. Avoid skin and eye contact – many amines are corrosive and can cause severe burns or irritation. Prevent inhalation of vapors, aerosols, or mists — use local exhaust ventilation at points of emission. Keep away from heat, sparks, and open flames, as many low-molecular-weight amines are flammable. Ensure compatibility of storage materials – amines may attack some plastics, rubber, or coatings. Store in tightly closed containers made of compatible materials (e.g. stainless steel, HDPE). Keep in a cool, dry, well-ventilated place away from direct sunlight. Segregate from: oxidizing agents (e.g. nitric acid, peroxides), acids (violent reactions possible, forming salts or heat), halogenated compounds and isocyanates (risk of exothermic reactions). Maintain secondary containment to prevent leaks or spills. | – | PPE, ventilation, leak prevention |
| End-of-life solvent management | X Disposal → Incineration – to prevent pollution. ☐ Energy recovery – using the heat from solvent incineration to produce steam or electricity. ☐ Reuse – through a regeneration process to remove contaminants. ☐ Landfilling – if the waste is non-hazardous or has been treated. ☐ Sale as raw material – supplying it to other facilities for use as feedstock in another process. | – | Recovery: only possible after purification (e.g. vacuum distillation) – rarely used. |

4. Are there any other factors that you consider as important?